# Wastewater Implementation Strategies

Wastewater management encompasses a broad range of efforts that promote effective and responsible water use, treatment, and disposal while encouraging the protection and restoration of the region’s — and this Project’s — watersheds. Properly designed, operated, and maintained sanitary sewer systems collect and transport all sewage that flows into them to a publicly owned treatment works (POTW). Wastewater treatment facility operators bear a large responsibility for converting the sewage into water that can be safely released back into the Trinity River. Table 8 lists the permitted WWTFs in the Greater Trinity Watershed. For the waste not handled as part of a sanitary sewer system, liquid waste haulers provide services to OSSFs and portable/chemical toilets. Given the bacteria-laden nature of wastewater (Lusk, 2011), broad attention in this I-Plan will be given to the wastewater system. WWTFs, sanitary sewer systems, lift stations, and liquid waste haulers all have the potential to impact bacteria loading in impaired waterways (see Implementation Strategies 5.0 – 5.5 for OSSFs).

## Implementation Strategies 1.0:  Wastewater treatment facility effluent limits

In November 2009, TCEQ commissioners approved Rule Project No. 2009-005-309-PR. This rule requires the addition of bacteria limits for E. coli in freshwater discharges for all TPDES domestic wastewater permits during their next permit amendment or revision. This rule is defined in Title 30 Administrative Code Chapter (TAC) 309.3(h) and the frequency of testing required is defined in 30 TAC Chapter 319.5(b). Through this control action, responsible entities will continue to monitor E. coli concentrations in WWTF effluent as required by individual WWTF permits and any subsequent permit amendments or revisions.

Currently, three permitted WWTFs (Table 8) have direct impact in the Greater Trinity Project area watershed(s) and three of those are currently required to monitor E. coli levels in their effluent. The remaining plant will be required to monitor for E. coli upon renewal of the permit. For TCEQ bacteria TMDLs in the Dallas-Fort Worth area, TPDES-permitted WWTFs are allocated a daily waste load allocation (WLAWWTF) calculated as their full permitted discharge flow rate multiplied by one half the instream geometric mean criterion. One-half of the water quality criterion (63 MPN/100mL) is used as the WWTF target to provide instream and downstream load capacity. Changes to effluent E. coli limits will occur following the approval of the TMDLs and during the next amendment or revision to an individual permit. Table 9 summarizes this implementation strategy.

Table 8. Permitted WWTFs in the Greater Trinity Watershed

| Facility Name | Permit Number | Permit Daily Average E. colia | Permit Effective Date | E. coli Permit Monitoring Frequency |
| --- | --- | --- | --- | --- |
| Dallas Central WWTF | WQ0010060-001 | 63 MPN/100 mL | 1/12/2017 | 5x/week |
| FTW Village Creek WWTF | WQ0010494-013 | 63 MPN/100 mLb | 1/3/2024 | 5x/week |
| TRA Central Regional WWTF | WQ0010303-001 | n/ac | 2/4/2008 | n/ac |

a There is also a daily maximum of 394 MPN/100mL.

b Subsequent renewals will include an E. coli limit of 63 MPN/100mL.

c Permit currently in renewal process. Renewed permit will include an E. coli limit of 126 MPN/100 mL and a monitoring frequency of 5x/week. Subsequent renewals will include an E. coli limit of 63 MPN/100 mL

Each of the entities listed in Table 8 is responsible for adhering to the requirements of their specific permits only. The terms and conditions in each individual permit are agreed upon by both the TCEQ and the permittee. Each permit specifically outlines the effluent constituents that require monitoring as well as the monitoring and reporting frequency to which the permittee must adhere. The TCEQ reviews and documents compliance with individual permits. WWTF permits are issued on a five-year cycle and must be renewed by the permittee. A map of WWTF coverage in the Project area can be found in Figure 7.
Table 9. Implementation Strategy 1.0 Summary — Wastewater treatment facility effluent limits

| Targeted Source(s) | WWTF effluent |
| --- | --- |
| Estimated Potential Load Reduction | Implementation Strategy (IS) 1.0 may result in a 2% reduction of calculated bacteria loading from WWTF effluent |
| Technical and Financial Assistance Needed | Technical: none — permit requirements are already being met Financial: none — permit requirements are already being met |
| Education Component | None |
| Schedule of Implementation | Immediate. New requirements for WWTF permits would come from TCEQ |
| Interim, Measurable Milestone | The number of permits requiring bacteria monitoring with reduced daily average limits |
| Progress Indicators | Allowable daily average will be reduced from 126 MPN/100 mL to no more than 63 MPN/100 mL for all WWTF discharging to impaired waterways |
| Monitoring Component | An annual report to Coordination Committee from NCTCOG to include information on the progress of implementation strategies, in addition to self-reporting by WWTF to TCEQ |
| Responsible Entity | WWTFs will meet permit requirements and monitor E. coli as appropriate NCTCOG will contact TCEQ to secure the necessary permit information pertaining to bacteria limits NCTCOG will provide Coordination Committee with information on WWTF effluent limits |

Figure 7. WWTF Location and Coverage Map with Permitted Dischargers

## Implementation Strategy 1.1:  Evaluation of non-participants in Sanitary Sewer Overflow Initiative (SSOI) and Capacity Management, Operation, and Maintenance (C-MOM) programs

Sanitary sewer systems that are properly designed, operated, and maintained will collect and transport all the sewage and industrial wastewater that flow into them to a wastewater treatment facility for appropriate treatment. If, however, there is significant inflow/infiltration (I/I) to the collection system; the system is not properly operated and maintained; or its capacity is inadequate, then sanitary sewers can overflow (Figure 8). The goals of the TCEQ SSOI are to reduce the number of sanitary sewer overflows (SSOs) that occur each year in Texas and to address SSOs before they harm human health, safety, or the environment and before they become enforcement issues (TCEQ, 2008).

Wastewater treatment facilities with sanitary sewer systems and subscribers within collection systems are eligible to participate in the TCEQ SSOI which provides benefits in that, a participating facility will not be subject to formal enforcement for most continuing SSO violations, as long as the SSOs are addressed by the SSO plan. Participation also allows the facility to spend resources on correction as opposed to having to pay penalties associated with an enforcement order, in addition to the money required to complete corrective action; and participation ensures that SSOs addressed by the SSO plan will not affect the facility’s compliance history rating.

C-MOM is a self-adopted program for owners and operators of sanitary sewer systems and involves proper management, operations, and maintenance of the collection system. Additionally, C-MOM programs ensure adequate capacity for peak flows, and take steps to prevent or mitigate SSOs.

Both SSOI and C-MOM programs have the potential to decrease bacteria loading by reducing SSOs. Table 10 lists SSOI participants and non-participants as of July 2019. As summarized in Table 11, the Coordination Committee or their appointees will evaluate the entities that do not participate in either the SSOI or C-MOM programs and as appropriate, encourage participation in one of those two programs.

Table 10. SSOI Participants

| Current and Past Participants as of 7/22/2019 | Not Currently Participating |
| --- | --- |
| City of Arlington | City of Cockrell Hill |
| City of Bedford | City of Colleyville |
| City of Dallas | City of Coppell |
| City of Euless | City of Dalworthington Gardens |
| City of Fort Worth | City of Haslet |
| City of Grand Prairie | City of Keller |
| City of Grapevine | City of Kennedale |
| City of Hurst | City of Mansfield |
| City of Irving | City of Richland Hills |
| City of North Richland Hills | City of Southlake |
| Trinity River Authority – Central WWTP System | City of University Park |
|  | Town of Highland Park |
|  | Town of Pantego |

Table 11. Implementation Strategy 1.1 Summary — Evaluation of non-participants in SSOI and C-MOM programs

| Targeted Source(s) | Sanitary sewer system (SSS) failures and SSOs |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.1, over 25 years, may result in a 35% reduction of calculated bacteria loading from SSSs and SSOs |
| Technical and Financial Assistance Needed | Technical: non-participants may need some level of technical assistance to begin SSOI and/or C-MOM participation Financial: grant funding, loans, and existing local funding as appropriate |
| Education Component | Outreach to SSS operators that are non-SSOI/non-C-MOM participants |
| Schedule of Implementation | Current: By 2018, all non-participating MS4s will have been contacted by Coordination Committee members, either as a whole or individually By 2028, SSOI/C-MOM participation will increase by 15% Revised: All non-participating MS4s will be a priority for contact by Coordination Committee members, either as a whole or individually With the goal of increasing SSOI/C-MOM participation by 15% annually or every 5 years |
| Interim, Measurable Milestone | Current: By 2018, 100% contact of non-participants Revised: Setting a goal of contacting x number of non-participants annually to completing a full cycle of contact by the 5-year re-evaluation point for the implementation status report. |
| Progress Indicators | The number of participants in SSOI and/or C-MOM |
| Monitoring Component | Current: An annual report to Coordination Committee from NCTCOG to include information on the progress of implementation strategies Revised: Every 5 years a report will be sent to the Coordination Committee from NCTCOG to include information on the progress of implementation strategies |
| Responsible Entity | NCTCOG will gather and distribute information about SSOI and C-MOM participation and use to the Coordination Committee The Wastewater subcommittee and Coordination Committee will conduct outreach to non-participants NCTCOG will contact TCEQ Office of Compliance and Enforcement Program Support Section annually to obtain a current list of SSOI participants for use in education and outreach efforts |

## Implementation Strategy 1.2:  Lift station evaluation

For a variety of reasons, lift stations may occasionally cease functioning and may discharge sewage into waterways. One example is lift stations ceasing to function during extensive power outages following severe weather. Lift stations may also fail to function during circumstances other than power outages, such as due to mechanical failure or during repair. However, unlike many SSOs, lift station failures can result in the discharge of large volumes of untreated wastewater into waterways.

The stakeholders encourage entities with lift stations to survey and evaluate existing stations by 2018 to determine the appropriateness of implementing best management practices (BMPs) to prevent SSOs caused by lift stations. Using this information, the Coordination Committee will re-evaluate the need for identifying or developing lift station BMPs for the BMP Library (see Implementation Strategy 8.0). Table 12 provides a summary of components necessary for lift station evaluation.

Table 12. Implementation Strategy 1.2 Summary—Lift station evaluation

| Targeted Source(s) | SSS failures and SSOs from lift station failures |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.2 may result in a 2% reduction in bacteria loading |
| Technical and Financial Assistance Needed | Technical: technical assistance may be necessary for lift station assessment and any potential repairs or alternations Financial: if technical assistance is not available internally to lift station owners and/or operators, then grant, loans, or local funding may be necessary for both evaluation and any potential repairs or alternations |
| Education Component | Outreach to SSS lift station operators |
| Schedule of Implementation | Current: By 2018, all entities with lift stations will have evaluated the need for maintenance programs to reduce SSOs caused by non or malfunctioning lift stations Revised: Every 5 years, all entities with lift stations will have evaluated the need for maintenance programs to reduce SSOs caused by non or malfunctioning lift stations |
| Interim, Measurable Milestone | None |
| Progress Indicators | Number of lift stations being evaluated by station owners and/or operators |
| Monitoring Component | Reports containing lift station owners and/or operators and their progress on evaluation will be made available to Wastewater technical subcommittee and Coordination Committee annually |
| Responsible Entity | Lift station owners and/or operators will evaluate lift stations and report progress to NCTCOG NCTCOG will report on progress indicator to the Wastewater technical subcommittee and Coordination Committee |

## Implementation Strategy 1.3:  Regional participation in Fats, Oils, and Grease program

Fats, oils, and grease (FOG) are considered the leading cause of blockages in sanitary sewers, and the EPA estimates that blockages account for nearly 50 percent of all SSOs (USEPA, 2007). North Texas Grease Abatement Council, now known since 2015 as the Wastewater And Treatment Education Roundtable (WATER), and NCTCOG have partnered to provide the cities and other agencies with public education materials related to FOG. Many organizations within the bacteria TMDL watersheds, such as Arlington, Colleyville, Dallas, Irving, North Richland Hills, and the TRA already use these materials to reduce FOG in the SSS and with it, SSOs. As summarized in Table 13, the stakeholders encourage organizations and wastewater plant operators to continue participation in the regional FOG education program. As resources are available, WATER is encouraged to expand educational materials to include the impact of FOG and SSOs on bacteria levels.

Table 13. Implementation Strategy 1.3 Summary — Regional participation in Fats, Oils, and Grease program

| Targeted Source(s) | SSO and SSS failures |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.3 may result in a 20% reduction in bacteria loading from SSOs and SSS failures |
| Technical and Financial Assistance Needed | Technical: technical assistance with FOG is available through existing programs Financial: participation in some FOG programs may require cost sharing, in addition to costs associated with educational materials; training for grease trap operators may also be necessary through grant funding, loans, and existing local funding as appropriate |
| Education Component | Outreach to RSWMP participants to ensure participation and outreach to non-RSWMP participants to encourage participation in regional FOG program(s) Public education is a primary component in FOG programs and an existing program is already in place Separate education programs may be necessary for grease trap operators |
| Schedule of Implementation | Current: Existing FOG public education participants will begin immediately and continue their programs as feasible. By 2018, outreach will be conducted to all MS4s with SSSs not participating in the regional FOG program Revised: Existing FOG public education participants will begin immediately and continue their programs as feasible. Outreach will be conducted to all MS4s with SSSs not participating in the regional FOG program every 5 years. As feasible have new drop off locations w/in TMDL project area to deal with FOG. |
| Interim, Measurable Milestone | Current: , all SSS owners and/or operators will actively participate in FOG programs Revised: Over 25 years, all SSS owners and/or operators will actively participate in FOG programs |
| Progress Indicators | Number of FOG program participants |
| Monitoring Component | NCTCOG will collect FOG participant information and report to Wastewater technical subcommittee and Coordination Committee |
| Responsible Entity | NCTCOG will gather and distribute information of FOG program participation and report results to the Coordination Committee and Wastewater technical subcommittee Wastewater technical subcommittee and Coordination Committee will conduct outreach to non-participants |

## Implementation Strategy 1.4:  Sanitary sewer overflow reporting

State law and TCEQ regulations specify reporting requirements for SSOs in Texas Water Code Chapter 26.039 and 30 TAC 305.125(9). Without accurate and available information on SSOs, gauging the effectiveness of SSO BMPs becomes difficult. Figure 8 provides a four-year representation of SSOs in the Project area categorized by the amount of released sewage. Table 14 summarizes the implementation strategies for SSOs.

### 1.4.1:  Wastewater and wastewater collection licensing

The Coordination Committee recommends TCEQ increase understanding of reporting requirements for SSOs and SSO mitigation by ensuring such information is included in wastewater licensing classes, including those for wastewater collection.

### 1.4.2:  Electronic reporting

The Coordination Committee encourages TCEQ to adopt electronic SSO reporting in addition to maintaining current methods. The TCEQ should further develop its system to allow electronic collection, analysis, and dissemination of this information. This action is not intended to increase the data-entry requirements for TCEQ staff; instead, it is intended to streamline reporting and analysis. Given technological disparities, however, the Committee encourages TCEQ to maintain the existing faxed SSO report for some time while electronic reporting is instituted.

### 1.4.3:  Reporting form changes

Current “source” descriptions on TCEQ’s reporting form are subject to interpretation. More accurate source descriptions would provide necessary information in future prevention of SSOs. TCEQ is encouraged to change the reporting form to better reflect actual cause of SSOs, for example specifying cause of blockage, and provide some type of education for those entities reporting.

Table 14. Implementation Strategy 1.4 Summary — Sanitary sewer overflow reporting

| Targeted Source(s) | SSOs |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.4. – 1.4.3 will contribute to the improved handling of SSOs and may result in a 2% reduction in calculated bacteria loading from SSOs over 25 years |
| Technical and Financial Assistance Needed | Technical: TCEQ may require technical assistance to develop appropriate database and reporting technologies as well as for wastewater licensing course materials SSS owners and/or operators may need high speed internet access or equivalent Financial: Existing and grant funding and loans as available |
| Education Component | TCEQ will provide appropriate instructions to SSS operators for using statewide SSO database TCEQ will provide appropriate educational materials for wastewater licensing course participants |
| Schedule of Implementation | As resources are available, the implementation of this activity will begin immediately and will continue for the entire implementation process |
| Interim, Measurable Milestone | Deployment of an appropriate database for tracking SSOs Wastewater licensing classes emphasizing accurate SSO reporting Reporting form changed for more accurate SSO cause description |
| Progress Indicators | Creation of a database Wastewater licensing course materials emphasizing SSO reporting Changed reporting form |
| Monitoring Component | NCTCOG will collect information from TCEQ regarding any updates to educational materials for wastewater licensing course participants, as well as any progress on database improvements |
| Responsible Entity | NCTCOG will coordinate with TCEQ on exploration of options for developing appropriate materials for use in wastewater licensing courses conducted through the TCEQ. NCTCOG will also coordinate with TCEQ to identify desired modifications to the SSO reporting form that would result in more effective SSO cause identification. SSS owners and/or operators will report SSOs as appropriate and ensure employee SSO reporting training NCTCOG will collect and share information with the Wastewater technical subcommittee and Coordination Committee |

## Implementation Strategy 1.5:  Funding opportunities for repair/replacement of sanitary sewer lines

Summarized below in Table 15, NCTCOG and stakeholders will pursue funding opportunities for rehabilitation or replacement of sanitary sewer lines, including Texas Water Development Board funding and regional supplemental environmental projects (SEPs) to repair, maintain, or extend wastewater infrastructure. NCTCOG will share information on funding opportunities to interested parties by web posting to a new or existing web page.

Table 15. Implementation Strategy 1.5 Summary — Funding opportunities for repair/replacement of sanitary sewer lines

| Targeted Source(s) | SSO and SSS failures |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.5 may result in a 5% reduction in calculated bacteria loading over 25 years by reducing the portion of the wasteload contributed by leaking or broken sewer lines |
| Technical and Financial Assistance Needed | Technical: engineering and technical expertise may be necessary Financial: existing or new grants, SEPs, or other funding mechanisms available at the local, state, or federal level |
| Education Component | NCTCOG will make new funding opportunities known to SSS owners and operators via web postings |
| Schedule of Implementation | As resources are available, the implementation of this activity will begin immediately and will continue for the entire implementation process |
| Interim, Measurable Milestone | Available funding opportunities identified on a NCTCOG web page |
| Progress Indicators | Creation of a new or modification of an existing web page for funding opportunities and the number of successful grant or funding applications for wastewater infrastructure received in the Project Area |
| Monitoring Component | Web page use reports for Coordination Committee and annual Water Quality Management Plan Update, which details some wastewater funding in the Project area |
| Responsible Entity | NCTCOG will create or modify existing web page and maintain current information SSS stakeholders will utilize information and seek funding opportunities to upgrade wastewater infrastructure |

## Implementation Strategy 1.6:  Relocation of sewer mains from waterways

Although waterways are convenient locations for sewer mains in terms of access rights and elevation, failures in the system in such locations have a direct impact on water quality and bacteria levels. The Coordination Committee encourages MS4s to relocate sewer mains out of waterways as practicable, as part of infrastructure replacement programs. Table 16 outlines the details of this implementation strategy.

Table 16. Implementation Strategy 1.6 Summary — Relocation of sewer mains from waterways

| Targeted Source(s) | SSO and SSS failures |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.6 may result in a 4% reduction over 25 years of calculated bacteria loading by reducing the potential for additional loading from leaking or collapsed sewer lines |
| Technical and Financial Assistance Needed | Technical: engineering and other technical expertise will be necessary in order to relocate wastewater lines from waterways Financial: grant funding, loans, and existing local funding as available |
| Education Component | Public education regarding relocation benefits may be needed Additionally, education for decision-makers, such as city councils, may also be necessary |
| Schedule of Implementation | Beginning immediately as appropriate, SSS owners and/or operators will consider relocation of sewer lines out of waterways as part of infrastructure repair and replacement |
| Interim, Measurable Milestone | Over 25 years, as many sewer lines as practicable will be relocated from waterways |
| Progress Indicators | Current: Number of sewer lines relocated Revised: Number of sewer lines relocated and routinely inspected |
| Monitoring Component | Voluntary reports from SSS owners and/or operators to NCTCOG on relocations |
| Responsible Entity | SSS owners and/or operators will relocate sewer mains from waterways as feasible |

## Implementation Strategy 1.7:  Liquid waste management and liquid waste hauler program expansion

Waste haulers routinely transport bacteria-laden materials, including septic, grease trap, and grit trap wastes. When this highly concentrated, untreated waste is discharged into waterways instead of being properly disposed of or treated, it may represent a significant local increase in bacterial loading.

NCTCOG and the Coordination Committee encourage MS4 permittees to maintain existing liquid waste hauler permit and inspection programs and expand them if necessary. Because liquid waste hauler regulation also takes place at the state level, the stakeholders request that TCEQ increase educational efforts to haulers, modify the registration form, and change regulations to include local notification. Table 17 summarizes the implementation strategies for liquid waste.

### 1.7.1:  Liquid waste hauler inspection program

Using sample ordinances available through the online BMP Library (see Implementation Strategy 8.0), municipal MS4s are encouraged to evaluate liquid waste hauler operations within their jurisdictions and create or expand inspection programs to include permitting, inspections, and tracking of liquid waste haulers; with a goal of having inspection programs in 100 percent of large MS4s by 2028 and 25 percent of small MS4s by 2033.

### 1.7.2:  TCEQ and liquid waste haulers

The Coordination Committee encourages TCEQ to increase its educational efforts toward liquid waste haulers, especially in regard to operations in areas with bacteria impaired waterways, illegal discharge penalties, and mitigation procedures.

#### 1.7.2.1:  Liquid waste hauler registration form addition

The Coordination Committee also requests TCEQ add a check box on liquid waste hauler registration forms for the operator to acknowledge that they know they are operating within an area with bacteria TMDL-listed waterways.

#### 1.7.2.2:  Requested change to liquid waste hauler regulations to include municipal notification

Request TCEQ amend regulatory guidance document to have waste haulers notify any municipalities, counties, and other jurisdictions that they are transporting through or where they are serving.

### 1.7.3:  Implementation of standards for portable/chemical toilets

MS4s are encouraged to implement standards concerning waste management on all sites requiring use of portable/chemical toilets to ensure placement as far from stormwater inlets, gutter lines, and water bodies as feasible and to ensure regular service scheduling of onsite waste facilities.

Table 17. Implementation Strategy 1.7 Summary — Liquid waste management and liquid waste hauler program expansion

| Targeted Source(s) | Improperly disposed waste from liquid waste haulers |
| --- | --- |
| Estimated Potential Load Reduction | IS 1.7 – 1.7.2 may result in a 5% reduction of calculated bacteria loading over 25 years by reducing the portion of the waste load contributed by improper handling, transportation, and disposal of liquid wastes |
| Technical and Financial Assistance Needed | Technical: some technical assistance may be necessary for MS4s without liquid waste hauler inspection and tracking programs to implement standards for portable and/or chemical toilets Financial: grants and/or existing funding and loans as available |
| Education Component | Outreach to MS4s without inspection and tracking programs may be necessary Educational efforts by TCEQ for liquid waste haulers regarding operations and any changes to registration form |
| Schedule of Implementation | Current: 100% of large MS4s will have inspection and tracking programs in place by 2028 25% of small MS4s will have inspection and tracking programs in place by 2033 Beginning immediately as feasible, TCEQ will consider changes to liquid waste hauler registration forms and changes to notification requirements. Revised: Large MS4s will have inspection and tracking programs in place as feasible and will be re-evaluated every 5 years. Small MS4s will have inspection and tracking programs in place as feasible and will be re-evaluated every 5 years. Beginning immediately as feasible, TCEQ will consider changes to liquid waste hauler registration forms and changes to notification requirements. |
| Interim, Measurable Milestone | Current: By 2028, 100% of large MS4s will have liquid waste hauler inspection and tracking programs in place by 2033, 25% of small MS4s will have liquid waste hauler inspection and tracking programs in place Revised: Large MS4s will have liquid waste hauler inspection and tracking programs in place as feasible and will be re-evaluated every 5 years. Small MS4s will have liquid waste hauler inspection and tracking programs in place as feasible and will be re-evaluated every 5 years. |
| Progress Indicators | Number of MS4s with inspection and tracking programs Number of MS4s with standards for portable and/or chemical toilets Changes to liquid waste hauler registration form(s) |
| Monitoring Component | Reports to Coordination Committee and Stormwater technical subcommittee regarding MS4 programs and TCEQ program/form changes for liquid waste haulers |
| Responsible Entity | MS4s will adopt liquid waste hauler inspection and tracking programs NCTCOG will coordinate with stakeholders and TCEQ staff to identify potential changes to the liquid waste hauler registration forms that will enhance their effectiveness. NCTCOG will compile information on programs and forms for annual report to Coordination Committee and Stormwater technical subcommittee |

Figure 8. Map — SSOs Occurring between January 2016 – December 2018

Other feedback from stakeholders:

- Update permit limits

- Include info on regionalization of ww

- Include info on Upper Trinity Water Quality Compact

- Update maps

- Perform analysis of load reductions – we identified what was needed, but how close are we to meeting those goals?